BY ORDER OF THE	AIR FORCE SYSTEMS SECURITY INSTRUCTION 3034

SECRETARY OF THE AIR FORCE	11 January 2000

Communications and Information

FORTEZZA USER REQUIREMENTS

COMPLIANCE WITH THIS PUBLICATION IS MANDATORY

NOTICE:  This publication is available digitally from the AF IP web page at:  http://www.afca.scott.af.mil/ip/.  If you lack access, contact your Publishing Distribution Office (PDO).

OPR:  HQ AFCA/GCIC (MSgt Cynthia Crowe)	           Certified By:  HQ AFCIC/SYIP (Lt. Col Pardo)

New Publication	   		           	Pages:  29

Distribution:  F

This instruction implements National Security Agency (NSA) Interim Operational Security Doctrine for the FORTEZZA Card, dated 18 February 1998, and NSA Interim Operational Security Doctrine for the FORTEZZA for Classified Card, dated 10 Jun 1998.  It provides guidance for the FORTEZZA Responsible Officers (FRO) and FORTEZZA users.  This instruction contains minimum security standards for the protection and use of the FORTEZZA user personal computer (PC) card.1  This instruction applies to all Air Force personnel, including civilians under contract by the Department of Defense, who use the KOV-11 and associated Public Key Infrastructure (PKI) X.509 version 1 certificates.  NOTE: This instruction applies to the FORTEZZA PC cards employing the Type 2 suite of algorithms only, and includes crypto cards and FORTEZZA-enabled PC modem cards.  Currently, this doctrine does not apply to PC cards employing a Type 1 suite of algorithms (e.g., FORTEZZA Plus).  As other FORTEZZA implementations evolve, this instruction will be updated to include appropriate controls.  When the term KOV-11 is used, it is referring to both the FORTEZZA card (used to protect sensitive information) and the FORTEZZA for Classified (FFC) card (used to protect SECRET information).  This AFSSI is authorized by AFI 33-206, Air Force Specialized Information Protection Publications.  Direct questions and comments on the contents of these instructions through command channels to Headquarters, Air Force Communications Agency, Information Protection (IP) Division (HQ AFCA/GCI), 203 W. Losey Street, Room 2040, Scott AFB IL 62225-5222.  Refer recommended changes and conflicts between this and other publications, using AF Form 847, Recommendation for Change of Publication, to HQ AFCA/GCIC.

SUMMARY OF REVISIONS

This document is new and must be completely reviewed.

1. FORTEZZA  is a registered trademark of the National Security Agency.

# Section A - General Information.

1.	Introduction.  This instruction sets procedures for FORTEZZA Responsible Officers (FRO) and FORTEZZA users.  It describes their duties and the minimum requirements for safeguarding, controlling, accounting for, and returning KOV-11s (FORTEZZA users' card).

1.1.	The combination of the KOV-11, FORTEZZA-enabled software and Public Key Infrastructure (PKI) provides the security services FORTEZZA offers.  Users must take the approved measures as outlined in this doctrine to maintain the security of both the KOV-11 and associated PKI.

1.2.	The KOV-11 provides cryptographic functionality and storage of keying material, authorization, and user identification.  When combined with FORTEZZA-enabled applications, the card provides confidentiality, user identification and authentication, and non-repudiation (proof of origin and receipt) in unclassified and classified environments.  NOTE:  Refer to paragraph 22 and 23 for the differences between individual and organizational certificates.

1.2.1.	Unclassified Environment.  The FORTEZZA card with FORTEZZA-enabled applications provides confidentiality, user identification, authentication, and non-repudiation sufficient to protect unclassified and/or sensitive information.

1.2.2.	Classified environment.  The FORTEZZA for Classified (FFC) is a FORTEZZA card programmed with PKI X.509 version 1 certificate(s) configured to protect classified information.  These “FFC certificates” are not classified in and of themselves.  The FFC card, with FORTEZZA-enabled applications, may be used on a secure network to provide user identification, authentication, non-repudiation, and secondary confidentiality of classified information if primary confidentiality of the network is ensured by a Type 1 product or a Protected Distribution System, or if the network exists completely within a secure enclave.  With the exception of the National Security Agency’s (NSA) Remote Access Security Program (RASP) Secret Dial-in implementation (see below), the FFC card with FORTEZZA-enabled applications alone shall not be used as the primary means of providing confidentiality for classified information in any medium.  NOTE:  A FORTEZZA card with FFC certificates configured for use in a SECRET environment is called an “FFC card with certificates for SECRET.”  A FORTEZZA card with FFC certificates configured for use in a TOP SECRET environment is called an “FFC card with certificates for TOP SECRET.”

1.2.3.	Remote User.  The FORTEZZA remote user capability allows a user outside a protected enclave, typically a traveling user with a laptop computer, to securely dial-in to that enclave over the Public Switched Telephone Network (PSTN).  The generic architecture includes a FORTEZZA PC card modem to establish the encrypted link, a FORTEZZA user card for digital signature and encryption, and a FORTEZZA-enabled media encryption application to protect the information resident on the remote user’s computer.  The remote user capability was designed to satisfy the limited requirement of providing security to traveling users who need temporary access to their home networks.  The NSA RASP Secret Dial-in Solution, which allows a remote user to securely access a protected enclave and exchange information classified up to Secret is described in Attachment 5.  NOTE:  The FFC card may be used to provide confidentiality, user identification, authentication, and non-repudiation for information up through SECRET over an unsecured medium only in the RASP Secret Dial-in solution (see Attachment 5) and only until January 1, 2002 (NSA mandated).

## Section B - Appointment of FORTEZZA Responsible Officer

2.	Appointing FORTEZZA Responsible Officers (FRO).  Unit Commanders (or equivalents) appoint, by letter, a primary FRO and at least one alternate to receive KOV-11s from the Certification Authority (CA), located within the COMSEC account (see attachment 2).  The letter includes each individual's name, rank, social security number (SSN), security clearance, and duty telephone.  Forward the original letter to the CA and a copy to the Sub-registration Authority (SRA).  The FRO will maintain a copy for his/her local files.  Update this letter at least annually or when changes occur.

2.1.	Persons appointed as primary FROs must have a minimum grade of staff sergeant (E-5), General Schedule (GS)-5, or DoD contractors of equal grade.  Alternates must have a minimum grade of Senior Airman (E-4) or GS-4.

2.2.	FROs must have at least a SECRET clearance.

2.3.	Commanders must also consider the following criteria when appointing a FRO:

2.3.1.	Units with an Established COMSEC Responsible Officer (CRO).  The CRO is an individual within an office or unit responsible for COMSEC material received from the COMSEC account.  CRO duties and appointment criteria are located in AFI 33-211, Communications Security (COMSEC) Users Requirements.  The CRO should also function as the FRO.

2.3.2.	Units with an Established STU-III Responsible Officer (SRO).  The SRO is an individual within an office or unit responsible for STU-III material received from the COMSEC account.  SRO duties and appointment criteria are located in AFI 33-209, Operational Instruction for the Secure Telephone Unit (STU-III) Type 1.  The SRO should also function as the FRO.

2.3.3.	Units with an Established Registration Authority (RA).  An RA is a local administrative authority who performs the registration function by gathering end-user registration information and forwarding it to the Certification Authority (CA).  AFI 33-127, Electronic Messaging Registration and Authority, lists of additional RA functions.  The RA should also function as the FRO.  NOTE:  Previous versions of documents used the term Organizational Registration Authority (ORA).  That term was dropped in favor of Registration Authority (RA) for consistency with other government documentation.

2.4.	Unit Commanders approve waivers for personnel with lower grades.  Waiver approval will be incorporated in a separate paragraph on the individual's appointment letter.

## Section C - Responsibilities

3.	COMSEC Manager.  The COMSEC Manager is responsible for all duties performed by the Certification Authority Workstation (CAW) Operators--the certification authority (CA), system administrator (SA), and information systems security officer (ISSO).  Specific duties and responsibilities for CAW Operations are defined in AFSSI-6001, Operational Instruction for Components and Systems Supporting the Multilevel Information Systems Security Initiative (MISSI).  The COMSEC manager:

3.1.	Ensures all programmed KOV-11s intended for use are issued to the appropriate FROs and are signed for by the FRO on the FRO Receipt Log.  An example of the FRO Receipt Log is located within AFSSI -6001.

3.2.	Ensures FROs are provided information on KOV-11 implementation, use, storage, restrictions, administrative procedures, transport, issue, controls, marking, accountability, reportable events, and physical security requirements before issuing KOV-11s.

3.3.	Provides FRO and all alternate FROs initial and refresher training according to paragraph 10.

3.4.	Ensures another FRO or alternate signs for KOV-11s, when a FRO transfers or if his/her security clearance access is suspended.

4.	Commander.  The commander of each unit that uses KOV-11s:

4.1.	Appoints, in writing, a FRO and at least one alternate to receive KOV-11s.  May appoint more than one FRO in large units, depending on the number of FORTEZZA users and the volume of KOV-11s handled.  Appoint FROs according to paragraph 2.

4.2.	Ensures approved security containers or facilities are available for storing FFC Personal Identification Numbers (PIN).

4.3.	Takes action on reportable events according to Section L.

4.4.	May appoint an Approving Official (AO) to approve all X.509 Certificate Request Forms used to create organizational certificates.  The AO maintains a list of current organizational office symbols authorized to perform organizational messaging with the privileges and clearance associated with each organization.  This is an optional appointment and commanders may combine this responsibility with other duties of the FRO.

5.	FORTEZZA Responsible Officers (FRO):

5.1.	Ensure all users granted access to FFC cards have proper clearance and a valid need-to-know.

5.2.	Conduct initial and refresher training of all FORTEZZA user according to paragraph 10 and attachment 3.

5.3.	Take responsibility for receiving, accounting for, handling, using, and safeguarding all KOV-11s until returned to the CA.

5.4.	Develop local operating instructions (OI) according to paragraph 9.

5.5.	Ensure X.509 Certificate Request Forms are properly completed, assisting users as needed, and verifying with the SRA that requested distinguished names (DN) are unique.

5.6.	Perform annual inventories of FFC cards, per paragraph 18.

5.7.	Issue KOV-11s and PINs to users.

5.8.	Return all KOV-11s as the CA directs.

5.9.	Maintain all records according to Air Force Manual (AFMAN) 37-139, Records Disposition-Standards (will convert to AFMAN 33-339).

5.10.	Ensure security checks are performed according to paragraph 12.

5.11.	Report all known or suspected events to the CA according to Section L.

5.12.	Coordinate with the CA the transfer of responsibilities to another FRO prior to leaving their current assignment or duties.

5.13.	Carry out additional duties requested by the COMSEC manager.

6.	FORTEZZA Users.  FORTEZZA users are responsible for safeguarding their KOV-11s in addition to the PIN numbers.  Ultimate success or failure rests with the individual users.  The careless user or the user who fails to follow procedures for using and safeguarding KOV-11s wastes all security efforts.  FORTEZZA users must make sure only authorized individuals have access to shared organizational cards.  Users must follow security rules at all times.  Report to the FRO or the COMSEC manager any circumstances or intentional or inadvertent acts which could lead to the disclosure of classified information, including its loss, improper use, unauthorized viewing, or any other instance that could possibly jeopardize the security of KOV-11 and the information it protects.  FORTEZZA users:

6.1.	Safeguard KOV-11s according to this document and control the cards locally.

6.2.	Return KOV-11s to the FROs on request, when no longer needed, and prior to permanent departure from the unit.

6.3.	Report immediately any reportable event of KOV-11s and their PINs to the FRO or CA according to Section K.

## Section D - Administrative Security Procedures

7.	FORTEZZA Specific Forms.  X.509 Certificate Request Form.

8.	Records Maintenance and Disposition.  Dispose of all records per AFMAN 37-139.

8.1.	Do not use correction fluid or correction tape on records that affect control and accountability of KOV-11s.  Use only original forms and complete using only blue or black ink.  For errors, neatly line through corrections, initial, and date.  Record explanatory remarks on the back of the form.  This includes the X.509 Certificate Request Form, FORTEZZA Receipt (Attachment 4), SF700, Security Container Information, SF701, Activity Security Checklist, and SF702, Security Container Check Sheet.

8.2.	Archive the User Advisory Statements and FORTEZZA Receipt (Attachment 4).  The original is retained by the CA and maintained according to AFMAN 37-139.  The CA determines if a copy is to be maintained by the FRO.

9.	Operating Instructions.  Each FRO must write an OI and coordinate it with the COMSEC manager.  The procedures and instructions in the OI are specific to the user's activity.  The OI includes procedures for handling, controlling, storing, transporting, and protecting the KOV-11s and PIN, as well as conducting an annual inventory, identifying training requirements, card rekey, access restriction, emergency procedures, reporting reportable events, user departures, remote users (if applicable), and the transfer of responsibilities to another FRO.

10.	Training.  Read and sign Attachment 3, FORTEZZA User Training, to document initial and refresher training for all persons with access to a KOV-11.  Provide annual refresher training, at least every 365 days, to all personnel who use KOV-11s, by having them read and sign attachment 3 annually.  Maintain only the most current documentation.

11.	Emergency Procedures.

11.1.	For fire, natural disaster, bomb threats, and covert threat, remove the KOV-11 from the terminal and secure it or keep it in the possession of authorized user until order returns.

11.2.	When evacuating the area, keep the KOV-11s in personal possession.

11.3.	For emergency destruction procedures for the KOV-11s, see paragraph 20.2.

12.	Security Checks.  Perform daily security checks at the end of each workday (or beginning of each shift for 24-hour operations) to ensure proper storage and accountability is maintained for all KOV-11s. Record the security checks on either the SF 701 (for daily operations) or DD Form 1753, Master Station Log (for 24-hour operations).  At a minimum, the security checks must include:

12.1.	Checking all card readers to ensure KOV-11s are removed.

12.2.	Making sure all KOV-11s are properly safeguarded according to paragraph 15.

12.3.	Making sure physical security systems or devices (for example, door locks, vent covers, etc.) work, and that safes, locking devices, outer doors, windows, and so forth, are locked.  If available, ensure alarms are set.

Section E - Security and Accountability.

13.	Card Classification.

13.1.	FORTEZZA.  The FORTEZZA card (used to protect sensitive information) is unclassified both when locked by a PIN (i.e., when not in use) and when unlocked (i.e., when in use).

13.2.	FFC.  The FFC card (used to protect SECRET information) is unclassified when locked by the PIN (i.e., when not in use).  When unlocked by the PIN (i.e., when in use), the FFC card is classified at the level of the highest classification asserted by certificates on the card.

13.2.1.	The implementing organization may, in order to further mitigate risk to its information, elect to handle the FFC card as classified material when not in use and locked with the PIN.  Any change from the default marking of UNCLASSIFED assigned to all user cards by the CAW (see paragraph 14.1.) would require that custom labeling be placed on the FFC card.  NOTE:  Paragraph 15.2.1 does not apply if the using organization decides to handle the card as a classified item.  If the FFC card is marked with the classification level, it must be protected at that level, whether it is locked or unlocked.

13.2.2.	An FFC card with both certificates for classified information from a classified CAW and certificates for unclassified information from an unclassified CAW is called a “dual-certificate” card.  A dual-certificate card is classified when unlocked and can only be used on a computer authorized to process classified information.

13.2.3.	Each user’s public key certificates for FFC use which are stored on the workstation, remain unclassified.

13.2.4.	Neither the FORTEZZA nor the FFC cards are to be marked or handled as CRYPTO (as defined in Attachment 1 - Terms).

14.	Card Nomenclature and Marking.

14.1.	Standard Marking.  The FORTEZZA user card is a commercial product available through the Federal Stock System and was assigned a federal nomenclature of KOV-11.  The FORTEZZA card and the FFC are identical except for the types of certificates loaded onto each, but both carry the nomenclature KOV-11.  The nomenclature KOV-11 is first physically applied to a card when it is programmed by the CAW.

14.2.	KOV-11 Label.  The CAW prints a card label that uniquely identifies each card.  This label contains the nomenclature (KOV-11), unique internal chip serial number, user name, card classification, and indicates if it is programmed with certificates for unclassified or classified information.  A card with FORTEZZA certificates (for unclassified information) has a white label, and an FFC card with certificates for SECRET information has a red label.  An FFC card with certificates for TOP SECRET information has a yellow label.  The user will not remove the card label.  Table 14.1 provides approved nomenclature and labeling guidance for FORTEZZA user cards.

14.3.	Obsolete Marking.  FORTEZZA nomenclature evolved during its deployment, so that many user cards deployed before 1999 are not consistent with paragraph 14.1.  KOV-11s programmed on CAWs operating Version 3.1 software do not print the label with the nomenclature as described in Table 14.1.  Instead, the label for a FORTEZZA user card does not include a nomenclature designation, while the FFC user card may be labeled “KOV-12,” with a white label printed in black type.  The CA may modify such labels with an appropriately colored indelible ink marker, or affix an additional label.  Replace the old label not in compliance with Table 14.1 when the user card is returned to the CA for rekey, but no later than December 31, 2004 (NSA mandated).

14.4.	FORTEZZA-enabled PC-card Modem.  The FORTEZZA-enabled PC-card modem (e.g., Palladium modem card) uses the same PKI certificates as a FORTEZZA card, and must have these certificates loaded by a CAW in the same manner as a standard FORTEZZA or FFC card.  The CAW treats the FORTEZZA modem card as a standard FORTEZZA card or FFC card, and so prints a label as discussed above.

| Table 14.1 FORTEZZA Card Information |  |  |  |  |
| --- | --- | --- | --- | --- |
| Type | Nomenclature | Label and Type Color | Classification when locked | Classification when unlocked |
| Organizational and personal FORTEZZA user card | KOV-11 | White/Black Type | Unclassified | Unclassified |
| Organizational and personal FFC user (with FFC certificates up through SECRET) | KOV-11 | Red/Black Type | Unclassified (See 15.2.1) | SECRET (See 15.2.2.) |
| Organizational and personal FFC user (for compartmented on TOP SECRET networks - see Attachment 5) | KOV-11 | Yellow/Black Type | Unclassified (See 15.2.1) | TOP SECRET (See 15.2.2) |

15.	Safeguarding the KOV-11.

15.1.	Unclassified.  Protect FORTEZZA cards (KOV-11s with only unclassified certificates loaded) in a manner similar to a credit card or high value item to limit the possibility of loss, unauthorized use, substitution, tampering or breakage.  Individuals may carry KOV-11s in their possession, provided the cards are given the level of protection discussed in this paragraph.

15.2.	FORTEZZA For Classified.

15.2.1.	Physically protect a locked FFC card (i.e., a card that is not in use) in a manner similar to a credit card or high value item to limit the possibility of loss, unauthorized use, substitution, tampering, or breakage.  Individuals may carry KOV-11s, provided the cards are given the level of protection discussed in this paragraph.

15.2.2.	An unlocked FFC card (i.e., an FFC card that is in use) must be afforded the protection commensurate with the certificates.  At no time will an FFC be left in an unattended terminal.  If the FFC card is not in active use, the user must log out of the application, remove the FFC card from its reader, and protect it as described in paragraph 15.2.1.

15.2.3.	An FFC card may be left unlocked in a continuously operating unstaffed infrastructure component (e.g. a Defense Message System Message Transfer Agent) provided the component or the space in which it is located provides protection commensurate with the classification of the information the component is processing.

15.3.	FORTEZZA PC Modem Card.  A FORTEZZA PC Modem card with certificates for unclassified information shall be afforded protection at least equal to that given the FORTEZZA user card.  Protect a FORTEZZA PC Modem card with certificates for classified according to paragraph 15.2.1.  However, since the FORTEZZA PC modem card has a much higher per unit cost, organizations using the FORTEZZA PC Modem card are encouraged to implement enhanced logistic controls.

16.	Safeguarding PINs.

16.1.	Memorize the PIN.  Do not write the PINs on the KOV-11 card or record in any manner in the vicinity of the host system for any reason.

16.2.	Do not share the PIN for individual cards.  Apply the requirements in paragraph 23.3, when sharing the PIN for organizational cards and duplicate cards for infrastructure components.

16.3.	The PIN for a KOV-11 is unclassified.

16.3.1.	For FORTEZZA cards, securely store the PIN or PIN letter physically separate from the card to prevent loss or unauthorized access.

16.3.2.	For FFC cards, record the PIN on an SF 700 and store in a GSA-approved container.  Destroy the PIN letter as classified waste.  An alternative to the SF 700 would be to store the PIN in a sealed envelope with initials over the envelope's seal in such a way that tampering will be obvious.

17.	Automatic Disabling.  The KOV-11 will disable itself after ten consecutive failed attempts to enter the PIN.  User’s must then contact their FRO (who will contact the CA) to arrange for card reactivation and new PIN assignments.  The CA will provide a new PIN and reactivate the card.  NOTE:  If the KOV-11 has several certificates, the first CA who programmed a certificate onto the card is the only one who can reactivate the card.

18.	Accountability.  Programmed KOV-11s are unclassified but locally accountable by the FRO to the CA.  Track and identify KOV-11s by means of the cards internal chip serial number and by the user's name, printed on the card label by the CAW as described above.  Do not track the KOV-11s by the manufacturer's external serial numbers, because these numbers are not unique.  The CA will affix the label identifying unique internal chip serial number and user name to the card.  Users will not remove the label.

18.1.	Inventory.  The FRO conducts a physical inventory all FFC cards (including duplicate cards) annually according to guidance provided by the CA.  The inventory must include physically viewing the card and rechecking the identity of the FFC owner.  FROs report the results of the inventory to the CA.  If an FFC card cannot be accounted for within 15 days of the required annual inventory, then the certificates associated with the missing FFC cards will be added to the Compromised Key List (CKL) and Certification Revocation List (CRL).  To avoid unnecessary revocation actions, ensure FFC cards are inventoried in a timely manner.

18.2.	Additional Accountability.  KOV-11s remain the property of the issuing organization.  Local policy may also require additional accountability of the cards.

18.3.	Change of FRO.  Prior to departure, the new FRO and the current FRO will jointly perform a physical inventory of all KOV-11s under their control.

19.	Classified Information.  Users must safeguard and control all classified information transmitted with the FFC card in accordance with AFI 31-401, Information Security Program Management.

20.	Disposal/Destruction.

20.1.	Routine Disposal.

20.1.1.	KOV-11.  Programmed KOV-11s that are excess or no longer required must be returned to the issuing CA who will determine if the card can be reused.  If cards are not returned in person to the CA, the cards must be returned via controlled methods (those methods which provide a continuous chain of accountability, as described in Section J.)

20.1.2.  PIN Letters.  Destroy the PIN letter as classified waste for FFC cards.  Destroy the PIN letters for FORTEZZA cards by tearing it up (minimum of at least 4 times), so no one can put it back together and throwing it into two separate trash containers.

20.2.  Emergency Destruction. If the capability exists and the time is available, return the KOV-11 to the CA who will zeroize as an alternative to physical destruction.  Destroy KOV-11s by smashing or breaking, ensuring the card is unusable.  Destroy organizational cards prior to individual user cards.

## Section F - Restrictions

21.	FORTEZZA and FFC Incompatibility.  Certificates loaded by an unclassified CAW are not cryptographically compatible with any certificates loaded by an FFC CAW.

22.	Individual Certificates.  KOV-11s with certificates programmed for individual messaging accounts must not be shared.  The authenticity of messages is based on individuals using only the card assigned to them.

23.	Organizational Certificates. KOV-11s with certificates programmed for organizational use (“organizational certificates”) carry additional requirements, as follows:

23.1.	Indicated on the X.509 Certificate Request Form, the primary individual who will control the KOV-11.

23.2.	The FRO must track the names of primary individuals to whom organizational certificates were issued.

| Figure 23.1 Example of Custody Log for Shared Organizational Cards |  |  |  |  |
| --- | --- | --- | --- | --- |
| Custody Log for KOV-11 |  |  |  |  |
| Internal Serial Number |  | Distinguished Name on Organization Card |  |  |
| 000000003ad4458d |  | 123ABW/FM |  |  |
| Name of User | Date/Time Issued | Initials | Date/Time Returned | Initials |
| TSgt Donald A. Davis | 3 Aug 99/0910 | DAD | 3 Aug 99/0945 | DAD |
| SSgt Martha O. Mathis | 4 Aug 99/1015 | MOM | 4 Aug 99/1056 | MOM |
| MSgt Kyle I. Donaldson | 4 Aug 99/1320 | KID | 4 Aug 99/1425 | KID |

23.3.	The individual, who signs for the shared organizational certificates, must ensure control of these certificates, including accounting for which individual had control of the keys at all times.

23.3.1.	A listing of who has access to the organization card must be created and maintained.  Limit the access list to only those users who have a need and the authority to send messages on behalf of the organization.

23.3.2.	A custody log (Figure 23.1) noting the KOV-11 serial number, the name of the user, date and the time issued and returned would meet the minimum requirement.

23.3.3.	For 24-hour operations, record the transfer of shared organizational cards from shift-to-shift on the DD Form 1753.

24.	Storing Certificates.  Storage on the KOV-11 is limited to certificates, keying material and security-critical information.  Users are not authorized to write any information to the KOV-11.

Section G - Control Requirements

25.	Access Controls.  The primary access control for the KOV-11 is having the proper security clearance, a need-to-know, supervisor's approval, and any other security standards imposed by the CA in order to issue a card and associated certificates to a given user.  The FRO will only issue FFC cards to properly cleared users.  The user must be cleared to a level equal to or greater than the card's highest certificate clearance.  NOTE:  This instruction refers only to granting access to a KOV-11.  All other requirements must be met prior to be granting access to a KOV-11.  Requirements such as foreign disclosure, investigation, security clearance, and foreign national access, still apply when granting access to the local area network (LAN) and the information contained in the systems connected to the LAN.

25.1.	U.S. Citizen user.  Users must demonstrate operational need for the KOV-11, as validated by their supervisor.  This also includes civilians under contract by the DoD.

25.2.	Non-U.S. Citizen user.

25.2.1.	Employed by the U.S. Government.  In certain situations, KOV-11s may be issued to permanently admitted resident aliens who are civilian employees of the U.S. Government, or are active duty or reserve component members of the U.S. Armed Forces.  The decision to issue a KOV-11 shall be made by the unit commander based on a determination that the official duties of the resident alien require this access.

25.2.2.	Employed by a U.S. Government Contractor or Vendor.  Permanently admitted resident aliens, and any other non-U.S. citizens, employed by a U.S. Government contractor or vendor may be issued KOV-11s only with the prior written approval of the appropriate Government Contracting Office.  Requests for KOV-11 issue must be fully justified and must be based on operational need.

25.2.3.	Foreign Nationals Employed by the U.S. Government.  Foreign Nationals who are resident aliens employed by the U.S. Government in support of operations may be issued KOV-11s, based upon a determination by the unit commander that the official duties of the Foreign National require the use of the KOV-11.

25.2.4.	Foreign Nationals Involved in Combined Exercises and Operations.  Foreign Nationals may be issued KOV-11s when combined military exercises and operations are initiated, and reclaimed when such operations are concluded.  Additional handling restrictions may be levied on foreign national users at the discretion of the issuing CA.  The decision to issue cards to Foreign Nationals shall be based upon an operational need determined by the cognizant U.S. military commander.

26.	Removing FFC Card from Classified Enclave.  Users who have an operational requirement to remove the FFC KOV-11 from a classified high enclave must be authorized by the cognizant Designated Approving Authority (DAA) and reminded of their responsibility to safeguard the card outside the secure enclave.  Users without such a requirement will satisfy paragraph 15.2. by securing their FFC KOV-11s within the secure enclave.  Users who have additional requirements to conduct unclassified transactions outside of an environment approved for classified information processing must be issued a separate unclassified-only KOV-11.  In general, do not remove the FFC card from the secure enclave except for administrative purposes (e.g., to transport card from issuing location to user’s location).

27.	Tampering.  Do not attempt to make a copy or tamper with the KOV-11.  It is the user's responsibility to report any suspected tampering attempts in accordance with Section K.  The user must not attempt any modification to the card’s software or hardware.

28.	Loaning Cards.  KOV-11s programmed and assigned to one individual user will not be loaned to any other individual.

29.	Inoperable KOV-11.

29.1.	Users must not attempt to repair or reprogram inoperable KOV-11.  An inoperable card must be returned to the issuing CA, who will determine if the card can be reused.

29.2.	If the KOV-11 cannot be returned in person to the CA, the cards must be returned via controlled methods that provide a continuous chain of accountability, as described in Section J.

29.3.	If the card itself is defective, a new card, new certificate, and new PIN will be programmed for the user.  The old certificates from inoperable KOV-11s must be revoked.  The user must notify all CAs who placed certificates on their inoperable KOV-11 so those certificates may be revoked.  To allow recovery of existing information (e.g., back traffic, text files, databases) encrypted with the old certificate, the old certificate will be reprogrammed onto the new card.  This will allow the user to recover and re-encrypt information from the old certificate to the new certificate.  The CA must revoke the old certificates 30 days after re-issue.

30.	Duplicate Cards.  Duplicate cards will only be created for infrastructure components (e.g. a Defense Message System Message Transfer Agent).

31.	Protecting Workstations.  Protect workstation(s) with FORTEZZA-enabled software in a manner sufficient to prevent loss, tampering, or unauthorized use of the system.  To prevent possible unauthorized use never leave a terminal unattended after the PIN has been successfully entered into the KOV-11 and the FORTEZZA-enable software is available for use.  An exception is allowed for continuously operating systems protected in accordance with paragraph 15.2.3.

31.1.	Lockscreens and screensavers are not acceptable means to protect an unlocked KOV-11.

31.2.	Remote workstation protection is addressed in Attachment 5.

## Section H - Information Systems Security

32.	Virus Detection.  Install AF-approved anti-virus software on workstations which run FORTEZZA-enabled applications.  Operate and update anti-virus software according to AFI 33-202, Computer Security.

33.	Configuration Management.  Use the most current version of FORTEZZA-enabled software.  If the user receives software and hardware changes, contact the local network system administrator to assist in confirming the authenticity and ensure the installation is approved by the DAA.

34.	FFC Architectural Approval.  Implementation of the FFC card, along with other necessary IAS/FFC components for protecting classified or compartmented information, must be approved by the appropriate DAA prior to use.

35.	Authorized Computers.

35.1.	FORTEZZA Card.  A FORTEZZA card and FORTEZZA-enabled software may only be used with a computer authorized to process unclassified or sensitive information.

35.2.	FFC Card.

35.2.1.	Use FFC cards with certificates for classified information only with a computer, and within an environment, that has been authorized for the processing of comparably classified and below information.  An FFC card with certificates for SECRET information, must be used on a system authorized to process SECRET and below information within a classified environment.  The only exception is the authorized FFC remote user (see attachment 5).

35.2.2.	Use FFC cards containing a separate certificate for TOP SECRET/sensitive compartmented information (TS/SCI) only on a computer authorized to process TS/SCI and below information only within a TS enclave.

35.2.3.	Use FFC cards with both (classified and unclassified) certificates from a classified CAW and unclassified certificates from an unclassified CAW (a “dual-certificate” card) only within a computer and an environment that is authorized to process information at the level of the highest classification asserted by certificates on the card.  Do not use an FFC card with dual-certificates in an unclassified computer/environment, except in approved remote user solutions (see attachment 5).

36.	Classification Labeling.  The FORTEZZA-enabled software (e.g., electronic messaging) must provide appropriate security labeling for the data being processed.  That is, the FORTEZZA-enabled software must require the user to insert the classification level of the data (message) and it must ensure the correct certificate is used based on the chosen classification and the cryptographic environment of the message recipient.

Section I - Public Key Infrastructure Requirements

37.	User Registration.  The prospective user must provide at least one official picture identification (e.g., military ID card, drivers license, etc.) to the FRO upon registration for the KOV-11.  See Section G for specific qualifications of a user.  The CA promulgates specific user registration procedures.

38.	Card and PIN Distribution.  Keep the KOV-11 and the associated PIN letter separate from the time they are produced by the CA until the intended user receives them.  The CA will issue KOV-11s and PINs to FROs.

38.1.	In the event of hand-delivery, and if the distribution facility is separate from the FRO’s working facility, the FRO may carry both KOV-11 and PIN letter back to the working facility.  Provide measures to minimize the risk of the loss or compromise of both card and PIN letter (e.g., don't carry card and PIN letter in the same container).  Seal the PIN in an envelope for delivery to the end user.

38.2.	The KOV-11 and associated PIN letter may be mailed to the user’s work address, provided the KOV-11 and PIN letter are mailed at least 3 days apart.  Mail cards according to Section J.

39.	User Advisory Statement and Receipt.  Upon receiving the KOV-11 and PIN, the user must sign and date a User Advisory Statement (Attachment 4), and return it to the FRO.  By signing the receipt, the user acknowledges receipt of both the KOV-11 and PIN, and the user agrees to accept the certificates listed on the documentation provided with the KOV-11.  If the user does not sign and return the User Advisory Statement to the FRO within 15 days, the FRO will notify the issuing CA who will revoke all of the user certificates on that KOV-11.

40. 	KOV-11 Rekey and Certificate Renewal.

40.1. 	KOV-11 Rekey.  Rekey the KOV-11 at least every 3 years.  Change the user's PIN when the card is rekeyed.  Return the KOV-11 to the issuing CA for rekey using transportation methods specified in Section J.

40.2.	Certificate Renewal.  Renew and validate the user's certificates at least every 3 years, normally when the card is rekeyed.  Also, renew the certificate to reflect any changes in the user's Distinguished Name as they occur.

40.3.	Notification.  Notification of pending key, PIN, or certificate expiration may occur in one of two ways:

40.3.1.	The FORTEZZA-enabled application or User Agent will alert to request rekey and certificate renewal from the issuing CA(s).

40.3.2.	The FRO will ensure user cards are rekeyed.  The FORTEZZA Receipt, signed by the user, lists the expiration date.

40.4.	The FRO will present the KOV-11s to the CA, who will reprogram the KOV-11s to renew all the certificates and key.  If the FRO fails to present the KOV-11s to the CA when notified to do so, the certificates will be added to the CRL within 72-hours and further use of the expired certificates will be inhibited.

41.	Loss/Compromise Requirements.  The loss or suspected compromise of a KOV-11 must be reported to the CA by the FRO no later than 1 working day after discovery.  Information required by the CA is described in Section K.  Users with KOV-11s programmed with certificates from multiple CAs must report to each CA.

42.	Certificate Revocation List (CRL) and Compromised Key List (CKL) Posting.  Many FORTEZZA applications automatically install the most recent CRL/CKL listing.  Contact the network system administrator for guidance regarding the user's specific platform to ensure the user's workstation always contains the most recent CRL/CKL listings.

43.	User Departure.  Prior to departing an organization, the user must return KOV-11s to the FRO who will give it to the CA.  Failure to return the KOV-11 will result in the CA reporting a compromise of user FORTEZZA keys to the Policy Creation Authority (PCA).

43.1.	Access to Organizational Cards.  Update the access list for the organizational card and remove the users upon departure or when access to the organization card is no longer needed.  Change the PIN number if the user will still have access to the LAN and the card.

44.	Digital Signature Verification.  The sender's digital signature prepared by the sender's KOV-11 must be verified to validate the authenticity of the transmitted data.  It is the responsibility of the receiving user to verify the content of any confirmation information presented by the application software.  If the FORTEZZA application software cannot verify the validity of the sender's signature, contact the sender and request a retransmission.  If, while verifying the authenticity of an incoming message, the FORTEZZA application reports that the sender's certificate has expired, do not accept the message as valid.

## Section J - Methods for Transporting Programmed KOV-11s

45.	Within U.S., its Territories, and Possessions.  To transport KOV-11 within the U.S., it territories and possessions, the organization/business transporting the KOV-11 must:

45.1.	Be incorporated in the U.S. and provide door-to-door service;

45.2.	Guarantee delivery within a reasonable number of days based on the distance to be traveled;

45.3.	Have a means of tracking individual packages within its system to the extent that, if a package becomes lost, the carrier can, within 24-hours following notification, provide information regarding the package's last known location;

45.4.	Guarantee the integrity of the transporting vehicle's contents at all times; and

45.5.	Guarantee the package will be afforded a reasonable degree of protection against theft (e.g., use of a security cage, video surveillance, etc.) if it becomes necessary for the carrier to make a prolonged stop at a carrier terminal.

46.	Outside the U.S., Its Territories, and Possessions.  To transport KOV-11 outside the U.S., it territories and possessions, the following applies:

46.1.	Its authorized user(s) may personally transport KOV-11s.

46.2.	Authorized U.S. Government department, Service or Agency couriers may transport KOV-11s.

46.3.	KOV-11s may be sent via U.S. Postal Service (USPS) registered mail provided the material does not, at any time, pass out of U.S. control, pass through any foreign postal system, or become subject to any foreign postal inspection.  USPS registered mail may be used to ship FFC cards to/from locations overseas, but only if the location is serviced by a Fleet Post Office (FPO) or Army Air Force Post Office (APO) that is authorized to process USPS registered mail.

46.4.	KOV-11s may be sent via U.S. military or military-contract air service carriers (e.g., AMC, Logair, Quicktrans) provided there is a continuous chain of accountability for the material while it is in transit.  Material shipped to/from locations outside the U.S., its territories and possessions must remain under continuous U.S. control.  Although some limited handling of the material by foreign nationals may be unavoidable during aircraft loading and unloading operations, the material must be returned to U.S. control upon completion of these operations.  If the material subsequently shows evidence of unauthorized access or tampering, report accordance to Section K of this document.

## Section K - Reportable Events

47.	Reportable Events.  Report the following events involving KOV-11 use to the FRO.  The FRO will report the information to the CA immediately, but no later than 1 working day after the event, for review and possible compromise recovery actions.

47.1.	Loss of Card.  The temporary or permanent loss of any KOV-11.

47.2.	PIN Compromise.  Actual or suspected compromise of PIN.

47.3.	Card Misuse.  Actual or suspected misuse of the KOV-11 and associated software (i.e., unauthorized modification to the FORTEZZA software installed on the host).

47.4.	Card Tampering.  Actual or suspected tampering with the KOV-11.

47.5.	Duplicate Cards.  Unauthorized use of an authorized duplicate KOV-11 or an unauthorized duplication of a KOV-11.

47.6.	Unreported Personal Data Changes.  User failure to notify the issuing CA of distinguished name (DN) changes.

47.7.	User Departure.  KOV-11 users leaving an organization without advising the FRO or CA concerning the status of their card.

47.8.	Premature Disabling.  Detection that a user’s card is disabled prior to the user making ten unsuccessful consecutive attempts to unlock their card.

47.9.	Incorrect Certificate Use.  The use of a sensitive certificate for protecting classified information.

47.10.	Unattended Terminal.  An unlocked FFC card left in an unattended terminal.

48.	Reportable Information.  The FRO will provide the following data to the CA to assess the impact of the events detailed in paragraph 47:

48.1.	User’s name, distinguished name, card serial number (chip internal serial number), and organization.

48.2.	All certificates and CAs who programmed certificates on the card.

48.3.	Complete circumstances of incidents, including physical security situations.

48.4.	Other personnel involved in the incident.

48.5.	What was potentially compromised (the card, PIN, or FORTEZZA software).

48.6.	User’s assessment of degree of potential compromise.

48.7.	Any additional information that may be requested.

Section K - Exceptions

49.  Exceptions.  Requests for exceptions to any of the provisions of this instruction must be submitted through Air Force Command Information Assurance (IA) channels to AFCA/GCI for analysis and AFCIC/SYI for approval prior to implementation.  All waiver requests for exceptions must be accompanied by complete operational justification.

|  | WILLIAM J. DONAHUE, Lt General, USAF Director, Communications and Information |
| --- | --- |

Attachment 1

GLOSSARY OF REFERENCES AND SUPPORTING INFORMATION

References

AFI 31-401, Information Security Program Management

AFI 33-127, Electronic Messaging Registration and Authority

AFI 33-202, Computer Security

AFI 33-209, Operational Instruction for the Secure Telephone Unit (STU-III) Type 1

AFI 33-211, Communications Security (COMSEC) User Requirements

AFSSI-6001, Operational Instruction for Components and Systems Supporting the Multilevel Information Systems Security Initiative (MISSI) (DRAFT)

NSA Interim Operational Security Doctrine for the FORTEZZA Card, dated 18 February 1998

NSA Interim Operational Security Doctrine for the FORTEZZA for Classified Card, dated 10 Jun 1998

Abbreviations and Acronyms

| ACL | Access Control List |
| --- | --- |
| AFB | Air Force Base |
| AFMAN | Air Force Manual |
| AMC | Air Mobility Command |
| AO | Approving Authority |
| APO | Army/Air Force Post Office |
| CA | Certification Authority |
| CAW | Certification Authority Workstation |
| CIC | Crypto Invocation Check |
| CKL | Compromised Key List |
| CMI | Certificate Management Infrastructure |
| COMSEC | Communications Security |
| CRL | Certificate Revocation List |
| CRO | COMSEC Responsible Officer |
| CSSO | Computer Systems Security Officer |
| DAA | Designated Approving Authority |
| DN | Distinguished Name |
| DoD | Department of Defense |
| FFC | FORTEZZA For Classified |
| FOUO | For Official Use Only |
| FPO | Fleet Post Office |
| FRO | FORTEZZA Responsible Officer |
| FTP | File Transfer Protocol |
| GS | General Schedule |
| HAG | High Assurance Guard |
| HQ AFCA | Headquarters, Air Force Communications Agency |
| IAS | Information Assurance Solutions |
| INFOSEC | Information Systems Security |
| IA | Information Assurance |
| IP | Information Protection |
| ISSO | Information System Security Officer |
| KMID | Key Material Identifiers |
| LAN | Local Area Network |
| MAJCOM | Major Command |
| MISSI | Multilevel Information Systems Security Initiative |
| NIPRNET | Unclassified But Sensitive Internet Protocol Router Network |
| NIST | National Institute of Standards and Technology |
| NSA | National Security Agency |
| NSI | National Security Information |
| NSTISSI | National Security Telecommunications and Information Systems Security Instruction |
| OI | Operating Instructions |
| ORA | Organizational Registration Authority |
| PAA | Policy Approving Authority |
| PC | Personal Computer formerly PC Memory Card International Association (PCMCIA) |
| PCA | Policy Creation Authority |
| PIN | Personal Identification Number |
| PKI | Public Key Infrastructure |
| PSTN | Public Switched Telephone Network |
| RA | Registration Authority |
| RASP | Remote Access Security Program |
| SA | System Administrator |
| SAP | Special Access Program |
| SAR | Special Access Required |
| SCI | Sensitive Compartmented Information |
| SF | Standard Form |
| SIPRNET | SECRET Internet Protocol Router Network |
| SRA | Sub-registration Authority |
| SRO | STU-III Responsible Officer |
| STU-III | Secure Telephone Unit III |
| TS/SCI | Top Secret/Sensitive Compartmented Information |
| U.S. | United States |
| U.S.C. | United States Code |
| USPS | U.S. Postal Service |

Terms

Certificate - A record holding security information about an information system user and vouches to the truth and accuracy of the information it contains.  A public key certificate contains the name of a user, the public key component of the user, and the identity of the user who vouches that the public key component is bound to the named user.  NOTE:  A public key certificate is normally issued to individuals.  When several individuals act in one capacity for an organization, an “organizational certificate” may be issued with the identity of the organization.

Certificate Revocation List (CRL) - A list of invalid certificates that have been revoked by the issuer.  It is periodically issued by each certification authority and posted to the directory.

Certification Authority (CA) - A person responsible for issuing and revoking user certificates, and exacting compliance with the PKI policy as defined by the parent Policy Creation Authority.  NOTE:  The term CA refers to both the authoritative office and role, and the incumbent in that office.  The CA is the third level in the PKI Certificate Management Infrastructure (CMI).

Certification Authority Workstation (CAW) - Commercial-off-the-shelf  (COTS) workstation with a trusted operating system and special-purpose application software.  The CAW programs KOV-11s with a user's security personality including certificates and cryptographic key.  The CA operates the CAW.

Compromised Key List (CKL) - A list generated by a Policy Creation Authority (PCA) that contains the Key Material Identifiers (KMIDs) of keys believed by the PCA to be compromised (i.e. that can no longer be trusted).

CRYPTO - Marking or designator identifying COMSEC keying material used to secure or authenticate telecommunications carrying classified or sensitive U.S. Government-derived information.

Designated Approving Authority (DAA) - Official with the authority to formally assume responsibility for operating a system at an acceptable level of risk.  This term is synonymous with designated accrediting authority, delegated accrediting authority, and information system security manager.

Directory - An on-line repository of certificates, CRLs, and CKLs.

Distinguished Name (DN) - An identifier, unique to a given PKI CMI that represents an individual’s identity.

Enclave - A general computing term used to describe an interconnected collection of some subset of an organization’s local computing resources.  An enclave is often comprised of heterogeneous platforms that operate in a multi-protocol environment sharing common characteristics.  These characteristics may include, but are not limited to common ownership/management, common mission, definable physical boundary, common security policy, controllable access point(s) to the enclave, and heterogeneous levels of trust.

FORTEZZA Card - A personal computer card that uses algorithms and procedures approved by National Institute of Standards and Technology (NIST) and the National Security Agency (NSA) to provide network related security services. Also identified as the KOV-11.  The card, when used in conjunction with the proper applications and network infrastructure, provides data integrity, access control, authentication, non-repudiation and confidentiality of user information.  NOTE: When the term KOV-11 is used, it is referring to both the FORTEZZA card (used to protect sensitive information) and the FORTEZZA for Classified (FFC) card.

FORTEZZA-enabled Application - Any software application that has been designed (or enabled) to use FORTEZZA cryptography when providing security services. This can be client software running locally on a user's workstation (e.g., File Transfer Protocol (FTP) clients, electronic mail clients, web browsers) or server software running unattended on a server (e.g., FTP servers, web servers, Directory System Agents).

Information Systems Security Officer (ISSO) - Person responsible to the designated approving authority for ensuring the security of an information system throughout its life cycle, from design through disposal.  Also know as the Computer Systems Security Officer (CSSO), within the Air Force.

Infrastructure Components - Components that provide connectivity to the backbone, provide management, control, security, and message transfer/conversion/expansion services to the site system and are required for the user components to function.

Locked - The KOV-11 is locked when it is not in a computer.

Key Material Identifier (KMID) - The KMID is a unique field contained in an X.509 certificate that identifies a specific set of private key material.  The KMID is unique within the Policy Approving Authority (PAA) hierarchy.

Need-to-know - The necessity for access to, or knowledge or possession of, specific information required to carry out official duties.

PC Card - Formerly known as Personal Computer Memory Card International Association (PCMCIA) cards.  The FORTEZZA cryptographic card is a peripheral that contains the processor, algorithms, and cryptographic material necessary to support personalized security services.

Personal Identification Number (PIN) - A random character string that unlocks and activates the KOV-11.

Public Key Infrastructure (PKI) - The framework established to issue, maintain, and revoke public key certificates.

Sensitive Information - Information, the loss, misuse, or unauthorized access to or modification of, which could adversely affect the national security interest or the conduct of federal programs, or the privacy to which individuals are entitled under Title 5 U.S.C. Section 552a (the Privacy Act), but that has not been specifically authorized under criteria established by an Executive Order or Act of Congress to be kept secret in the interest of national defense or foreign policy.  The responsibility for determining the applicability of the handling caveat or warning notice lies with the originating organization.  (Systems that are not national security systems, but contain sensitive information are to be protected in accordance with the requirements of Public Law 100-235, the Computer Security Act of 1987).  NOTE:  The MISSI program formerly referred to this information as “sensitive-but-unclassified” (sensitive).

System Administrator (SA) - Individual responsible for the installation and maintenance of an information system, providing effective information system utilization, adequate security parameters, and sound implementation of established Information Assurance policy and procedures.

Type 1 Product - Classified or controlled cryptographic item endorsed by the NSA for securing classified and sensitive U.S. Government information, when appropriately keyed.  The term refers only products, and not to information, key, services, or controls.  Type 1 products contain classified NSA algorithms.  They are available to U.S. Government users, their contractors, and federally sponsored non-U.S. Government activities subject to export restriction in accordance with International Traffic in Arms Regulation.

Type 2 Product - Unclassified cryptographic equipment, assembly, or component, endorsed by the NSA, for use in national security systems as defined in Title 40 U.S.C. Section 1452.

Unlocked - The KOV-11 is unlocked when it is in a computer and the PIN entered (activated).

Attachment 2

EXAMPLE OF APPOINTMENT LETTER FOR
FORTEZZA RESPONSIBLE OFFICER (FRO) AND ALTERNATES

(Letterhead)

Date

MEMORANDUM FOR (Wing COMSEC Account)

FROM:  (Office and Address)

SUBJECT:  Appointment of FORTEZZA Responsible Officer (FRO) and Alternate(s)

1.  The following persons are assigned as Primary FRO and Alternate FRO(s) for (unit/office symbol).  They may receipt for KOV-11s up to and including (clearance) from (COMSEC Account Number).

PRIMARY FRO

NAME:

RANK:

SSN:	CLEARANCE:

PHONE:

E-MAIL ADDRESS:

ALTERNATE FRO(s)     (repeat this information for all alternates)

NAME:

RANK:

SSN:	CLEARANCE:

PHONE:

E-MAIL ADDRESS:

2.  Request you train the appointed FRO and alternate FRO(s) on the handling and safeguarding of KOV-11s according to AFSSI 3034.

3.  This letter contains personal information protected by the Privacy Act of 1974.

4.  This letter supersedes all previous letters from this office on this subject (or give specific dates).

(Commander’s Signature Block)

Attachment 3

FORTEZZA USER TRAINING

NOTE:  When the term KOV-11 is used, it is referring to both the FORTEZZA card (used to protect sensitive information) and the FORTEZZA for Classified (FFC) card (used to protect SECRET information).

1.  A locked KOV-11 (i.e., a card that is not in use) is considered unclassified.  When the KOV-11 is in the terminal in the unlocked mode (KOV-11 in the terminal and PIN activated), afford protection commensurate with the highest level of the key it contains and ensure use only by the authorized individual.

2.  When unauthorized personnel are in the area, discontinue the processing of sensitive or classified information on the terminal.  Pay strict attention to the authorized classification level of the KOV-11 and ensure transmission does not exceed the highest authorized classification.  To prevent possible unauthorized use never leave a terminal unattended after the PIN has been successfully entered into the KOV-11 and the FORTEZZA-enabled software available for use.  Lockscreens and screensavers are not acceptable means to protect an unlocked KOV-11.  Do not leave a FFC card in a terminal unattended.

3.  When not in use, protect KOV-11s in a manner similar to a credit card or high value item to limit the possibility of loss, unauthorized use, substitution, tampering, or breakage.  The KOV-11 must be in the user’s possession at all times.  When sleeping, protect the KOV-11 as you would your money, credit cards, and other valuables.

4.  Memorize the PIN.  Do not write the PIN on the KOV-11 card or record it in any manner in the vicinity of the host system for any reason.  Do not share the PIN for individual cards.  The PIN for a KOV-11 is unclassified.  For FORTEZZA cards, securely store the PIN or PIN letter separate from the card to prevent loss or unauthorized access.  For FFC cards, record the PIN on an SF 700, Security Container Information, and store in a GSA-approved container.  Destroy the PIN letter as classified waste.  An alternative to the SF 700 would be to store the PIN in a sealed opaque envelope with initials over the envelope's seal in such a way that tampering will be obvious.

5.  Maintain an access list of authorized users of the organizational card.  A custody log for the KOV-11 will be maintained when sharing the PIN for organizational cards and duplicate cards for infrastructure components.

6.  Users are responsible for the proper safekeeping and immediate reporting of its loss, destruction, theft, or tampering.  Notify your FRO immediately if an FFC cards is left in an unattended terminal or a KOV-11 or its PIN is compromised.

7.  In the event of fire, natural disaster, or covert threat, remove the KOV-11 from the terminal and secure it or keep it in the possession of the authorized user.

| ______________________________ | ______________________________ |
| --- | --- |
| Printed/Typed Name of User | Signature of User |
| ______________________________ | ______________________________ |
| Printed/Typed Name of Trainer | Signature of Trainer |

(Modify this training list for your location)

Attachment 4

FORTEZZA USER ADVISORY STATEMENT AND RECEIPT

The KOV-11, FORTEZZA®/FFC cryptographic card, is a self-contained security device which provides you, the user, with a digital certificate - a unique electronic "personality" - and all the cryptographic support functions you need to perform electronic digital signature, encryption, and decryption of your information, all in a flexible and compact package.

Because of its compact size, the cryptographic card can be carried readily in a pocket or purse.  Used properly, your KOV-11 will provide you with a high degree of security.  Be aware that loss of your KOV-11 may place the information you have protected at risk.  A hostile entity that has unauthorized access to your card could attempt to use the card to decrypt information protected by that card.  Furthermore, it could enable an unauthorized person to electronically masquerade as you.

While the KOV-11 does employ a Personal Identification Number (PIN) to prevent use of the card by unauthorized parties, no PIN-based system is absolutely foolproof.  You must, therefore, take precautions to protect your KOV-11 and PIN from loss or theft.

Immediately report loss, attempted theft, destruction, tampering, or any similar possible compromise of your KOV-11 to your FORTEZZA Responsible Officer (FRO) at __________ or Certification Authority (CA) at __________.

By signing and returning this form to your FRO, you agree to the following terms:

"I have read this statement and acknowledge receipt of the KOV-11 identified by serial number below, the associated PIN, and certificate noted on the PIN letter.  I also agree to follow AFSSI 3034, FORTEZZA User Requirements, pertaining to the policy for the KOV-11s."

| - Card Type | - Individual or Organizational |
| --- | --- |
| - Card Serial Number: |  |
| - Expiration Date: |  |
| - Unit/Office Symbol: |  |
| - Duty Phone Number: |  |
| - Name of User: |  |
| - Signature: |  |
| - Date of Receipt: |  |

Attachment 5

USE OF THE FFC CARD BY A REMOTE USER

A5.1. Capability.  The FORTEZZA remote user capability allows a user outside a protected enclave, typically a traveling user with a laptop computer, to securely dial-in over the Public Switched Telephone Network (PSTN).  The remote user capability was designed to satisfy the limited requirement by providing security to traveling users who need temporary access to their home networks.

A5.2.  RASP Secret Dial-in Solution.  The NSA Remote Access Security Program (RASP) Secret Dial-in Solution uses a KOV-11 with certificates for SECRET information and the Palladium Modem Card (a FORTEZZA-enabled PC-card modem in a remote access server at the protected enclave boundary.  The FORTEZZA-enabled modem cards are also programmed with FFC certificates for SECRET information.  The third distinct component of the RASP Secret Dial-in Solution is a FORTEZZA-enabled media encryption application.

NOTE:  The FORTEZZA-enabled PC-Card modem may be loaded either with certificates for classified information, or with certificates for unclassified information.  When loaded with certificates for classified information, the PC-card modem supports secure remote access to a classified enclave.  When loaded with certificates for unclassified information, the PC-card modem supports secure remote access to a sensitive (unclassified) enclave.

A5.3.  Restrictions.

A5.3.1.  Remote users must be approved in writing by their commander to process classified information (up to SECRET) outside a formally approved facility.  The overall implementation of this capability must be approved by the DAA (see paragraph 26).

A5.3.2.  Remote user capability must be limited to only those users having an operational requirement to process sensitive through classified (up to SECRET) information outside of the protected enclave.

A5.3.3.  Only direct-dial access to the protected enclave is permitted; access through unclassified, non-secure networks such as commercial Internet service providers is not permitted.

A5.3.4.  Remote users must use FORTEZZA-enabled devices to access the classified enclave through a boundary device.

A5.4.  Physical Security.  Physical security is of paramount concern in the remote user environment.  The user must be aware of his or her surroundings at all times when processing classified or sensitive information outside of a protected enclave.

A5.4.1.  The remote user must keep the KOV-11 on his/her person or stored according to paragraph 15.2.1.

A5.4.2.  The KOV-11 and FORTEZZA-enabled PC-Card modem must be kept separate from the computer when not in use, and must not be stored or carried in the same bag with the laptop.

A5.4.3.  The KOV-11 and computer are classified only when the KOV-11 has been inserted and unlocked by the PIN (except as noted in A5.4.7. below).

A5.4.4.  All information files, classified and unclassified (e.g., electronic message files, document files, graphic or image files), stored on the classified hard drive of the computer used for remote operations, must be encrypted by the KOV-11 using a FORTEZZA-enabled media encryption program.

A5.4.5.  The hard drive used for FORTEZZA-encrypted classified processing is not classified itself, since the information is fully encrypted, yet it must be kept distinct from the non-FORTEZZA hard drive and provided more protection.  Remote users must mark the hard drives in accordance with cognizant DAA policy to distinguish the hard drive used for FORTEZZA encrypted classified processing from the hard drive used for unclassified processing.

A5.4.6.  When not in use and when media encryption is employed, the computer is considered unclassified, but must be physically protected by either being in the personal possession of the authorized, cleared user or stored minimizing the possibility of loss, unauthorized use or tampering.

A5.4.7.  If a FORTEZZA-enabled media encryption application is not available for the user’s computer, then the computer must be physically protected commensurate with the classification of information processed by the PC.

A5.5.  Unclassified Certificate Limitations.  Classified information must not be transmitted using the unclassified certificates (which are designated solely for unclassified applications).  Report violations of this nature in accordance with AFI 31-401.

A5.6.  Organizational Certificates.  The FORTEZZA-enabled PC-card modem (e.g., the Palladium secure modem is a relatively high-cost item, and may be loaded with organizational certificates to facilitate sharing.  FORTEZZA-enabled PC-card modems with organizational certificates shall be controlled in accordance with paragraph 18 of this instruction.

A5.7.  Connections to Non-FORTEZZA Applications.  All remote users using an FFC application are prohibited from intentionally communicating with external non-FORTEZZA applications while the same hard drive used for FORTEZZA applications is integrated with the computer.  If direct connectivity to unclassified network services on the Internet (including using Internet service providers such as CompuServe or America On-Line) is needed, the hard drive to the computer must be temporarily replaced with another drive to be used only for these applications.  Additionally, users must cycle (turn off power for a minimum of five seconds) the computer power before any drive replacement.

Attachment 6

USE OF THE FFC CARD TO SEPARATE COMPARTMENTED DATA

A6.1.  TS/SCI Network.  The FFC card may be used to separate sensitive compartmented information (SCI) (including various releasability caveats), and Special Access Program/Special Access Required (SAP/SAR) information on a TS/SCI approved system or network.  The application of FFC to protect SCI or SAP/SAR on a TS/SCI system must be approved by the DAA for the TS/SCI system.  A central Access Control List (ACL) for the compartment is established and maintained by the compartment or program’s Control Officer.  The FORTEZZA provides Identification and Authentication of the individual users who are authorized access to the compartmented program.  This technique employs the unique electronic “personality” (i.e., certificate) of the user stored in the FORTEZZA card as a positive method of identification.  Access to SCI or SAP/SAR is enforced by either:

A6.1.1.  Using FORTEZZA identification and authentication to verify a user against the ACL prior to allowing the user access to SCI or SAP/SAR information, or;

A6.1.2.  Having the sender and receiver of SCI or SAP/SAR information verify each other’s identity against the ACL prior to exchanging the information.

A6.2.  SECRET Network.  The FFC card may be used to enforce releasability caveats and need to know within a SECRET approved system or network (e.g., SIPRNET).  This application of FFC must be approved by the DAA for the system.  As with the TS/SCI compartment scenario, a central Access Control List (ACL) for the caveat or need to know control is established and maintained by the cognizant authority.  The FORTEZZA provides Identification and Authentication of the individual users who are authorized access to the information.  Access to the need-to-know information is enforced by either:

A6.2.1.  Using FORTEZZA identification and authentication to verify a user against the ACL prior to allowing the user access to the information, or;

A6.2.2.  Having the sender and receiver of compartmented information verify each other’s identity against the ACL prior to exchanging the need-to-know information.